Electric Circuits (EE 100)	Lab Manual Handout # 7

| Name | Muhamad Asad |
| --- | --- |
| Reg. No | 2019-EE-383 |
| Marks / Grade |  |

# EXPERIMENT NO. 7

THE NODE-VOLTAGE AND MESH-CURRENT METHOD

## Objective:

To study the validity of the Node-voltage and the Mesh-current method.

## Apparatus:

- Resistors

- Digital MM

- Breadboard

- DC power supply

- Connecting wires

## Pre-requisite

Before coming to the lab, students must study and practice the node-voltage and mesh-current methods. Also review the theoretical concepts relating Kirchhoff’s voltage and current laws.

Safety Precautions

- Look at each exercise carefully before connecting the circuits.

- Make sure all power is off before connecting or disconnecting components.

- Ask your Instructor to check the circuit before turning on the power.

- When measuring voltage or current, make sure the DMM is correctly set for what you need to measure.

Electric Circuits (EE 100)	Lab Manual Handout # 7

## Procedure:

PART 1: NODE-VOLTAGE METHOD

The node-voltage method uses KCL equations that are written at all non- reference nodes. Recall that KCL states that the sum of all the currents at a node is zero. The node- voltage method is comprised of the following steps:

- Identify all the nodes in a circuit.

- Choose one of the nodes as the reference node (usually a low potential node).

- Assign variable names to each of the essential (non-reference) nodes. Mark voltages as per the variable names assigned to each node e.g. va, vb, vc, etc.

- Determine the direction of currents i.e. mark voltage polarities across each circuit element.

- Write a KCL equation in terms of node-voltages at each of the non-reference nodes. The result will be (n-1) equations where n is the number  of  essential  nodes  plus  a  reference node.

- Solve the equations and verify values through measurements.

## Circuit Diagram:

a

R4

b

c

1

R1

R3

R2

5V

10V

Figure 7.1

## Calculation & Observation:

| Resistance values | Calculation results |  | Measurement results |  |
| --- | --- | --- | --- | --- |
|  | Voltage | Current | Voltage | Current |
| R1=3 | 14.33 | 4.76 |  |  |
| R2=5 | -4.333 | -0.866 |  |  |
| R3=7 | -9.33 | 1.33 |  |  |
| R4=9 | 5 | 0.555 |  |  |

### Note: In the calculations, first write the equation for all the currents in terms of node voltages and resistances. For example, in fig. 7.1, the current flowing through R2 is Vb/R2.

PART 2: MESH-CURRENT METHOD

The mesh-current method, like the node-voltage method, provides a systematic means to specify the equations needed to solve a circuit. The method uses the meshes in a circuit. A mesh is a loop in the circuit that does not contain any other loops within it. The mesh-current method uses KVL equations that are written for each mesh in the circuit. Recall that KVL states that the sum  of  all  the  voltages drops and rises around a closed path (or loop) is zero. The mesh-current method can be broken into the following steps:

- Identify all the meshes in a circuit. To do this draw a curved arrow to identify the direction (clockwise, conventionally) of the current flowing in the mesh.

- Assign variable names to each of the meshes. Mark currents as per the variable names assigned to each mesh e.g. Ia, Ib, Ic, etc.

- Write a KVL equation in terms of mesh-currents for each of the meshes in the circuit. The result will be n equations where n is the number of meshes.

- Transform all the equations into standard form. The resultant equations can be solved using matrix method or any other method of your choice.

- Solve the equations for the currents. Find voltage across each component using Ohms law. Finally verify the values through measurements and fill in the table given below.

Figure 7.2: Example circuit to test the validity of the mesh-current method

## Calculation & Observations

| Resistance values | Calculation results |  | Measurement results |  |
| --- | --- | --- | --- | --- |
|  | Voltage | Current | Voltage | Current |
| R1 =3 | 2.0625 | 0.68 |  |  |
| R2 =5 | 0.5 | -0.1 |  |  |
| R3 =7 | 1.43 | 0.217 |  |  |
| R4 =9 | 1.677 | 0.64 |  |  |
| R5 =11 | 3.1 | 1.1 |  |  |
| R6 =13 | 0.8 | 0.1 |  |  |

### Note: In the calculations, also write the equation for all the currents in terms of resistances.

Task: