Electric Circuits (EE-100)	Lab Manual Handout # 9

| Name | Muhammad Asad |
| --- | --- |
| Reg. No | 2019-EE-383 |
| Marks / Grade |  |

EXPERIMENT NO. 9 THEVENIN THEOREM

# Objectives:

- Verify the Thevenin theorem through an experiment.

- Find the Thevenin’s resistance RTH by various methods and compare values.

# Equipment:

Resistors DMM

Breadboard

DC power supply Connecting wires.

# Prerequisite:

Before coming to the lab, students must study and practice  the  Thevenin theorem,  and also review other circuit analysis techniques.

# Theory Overview:

Thevenin theorem states that any linear two-terminal circuit (Fig. 9.1a) can be replaced by an equivalent circuit (Fig. 9.1b) consisting of a voltage source VTH in series with a resistance RTH where

- VTH is the open-circuit voltage at the terminals, and

- RTH is the equivalent resistance seen through the terminals provided that all the independent sources are turned off.

Electric Circuits (EE-100)	Lab Manual Handout # 9

Linear two- terminal circuit

Load

Load

Figure 9.1a: A linear two-terminal circuit	Figure 9.1b: Thevenin equivalent circuit

Finding VTH: Determine the output voltage Vab in open-circuit condition (no load resistor meaning infinite resistance). This is known as the Thevenin voltage VTH.

i.e. VTH = Vab = VOC (open-circuit voltage)

+

V

-

a

Circuit

b

Voltmeter

Electric Circuits (EE-100)	Lab Manual Handout # 9

Finding RTH: There are different methods to find value of RTH as described below.

- Method A: (By measuring ISC – short-circuit current)

Short the points ‘a’ and ‘b’, then measure the short-circuit current ISC using an ammeter. Calculate RTH as follows,

RTH  = VTH / ISC	(9.1)

ISC

A

b

Circuit

a

Ammeter

- Method B: (By measuring the equivalent resistance)

Remove all sources from the circuit, i.e. replace all voltage sources with a short- circuit and current sources with an open-circuit. Then with the help of a multimeter find the resistance between the points ‘a’ and ‘b’, denoted by Req.

RTH = Req	(9.2)

Req

Ω

b

Turn-off all

the sources

a

Multimeter

- Method C: (By applying known voltage and measuring source current)

We can also find RTH by applying a known voltage to the circuit between points ‘a’ and ‘b’, then measuring the current from the voltage source. Suppose Vtest = 1 V is applied as the test voltage then,

RTH = Vtest / I	(9.3)

Ammeter

I

a

Turn-off all

the sources

Req

b

A

+-

Vtest

Electric Circuits (EE-100)	Lab Manual Handout # 9

- Method D: (By inserting two different resistors and measuring current)

There is another method to find the value of RTH, by inserting two different  loads between points ‘a’ and ‘b’ i.e. insert resistors R11 and then R22 one by one and write expression for the current through that particular resistor as given below,

R11

b

Circuit

a

Current through the resistor R11 :-	I1

I1 = VTH / (RTH  + R11)	(9.4)

R22

b

Circuit

a

Current through the resistor R22 :-	I2

I2 = VTH / (RTH + R22)	(9.5)

Simultaneously solving equations (9.4)-(9.5) will give the value of RTH.

# Procedure:

  - Connect on breadboard the circuit shown in Fig. 9.2.

  - The aim is to determine the current through the 1kΩ resistance R1 connected between the two nodes A and B.

  - The voltage across the 1kΩ resistance R1 connected between the two nodes A and B.

Safety Precautions

- Look at each exercise carefully before connecting the circuits.

- Make sure all power is off before connecting or disconnecting components.

- Ask your TA to check the circuit before turning on the power.

- When measuring voltage or current, make sure the DMM is correctly set for what you need to measure.

Follow the following procedure to validate the Thevenin’s theorem.

Electric Circuits (EE-100)	Lab Manual Handout # 9

Finding VTH:

- Remove the resistor R1 from the circuit on breadboard.

- Determine the open-circuit voltage (voltage between points A and B i.e. VAB) using       a voltmeter. This is the required Thevenin voltage VTH. Record the value in Table 9.1.

R1

V1

C

R2

D

10Vdc

R3

E

A	 R5	B	R4

V2

5Vdc

0

Figure 9.2: Example circuit to test the validity of the Thevenin’s theorem

R1

V1

C

R2

D

10Vdc

R3

E

A	B	R4

V2

5Vdc

0

Figure 9.3: Finding VTH